STANDARD OPERATING PROCEDURE

# Replace the gripper servo (motor 6) on the SO-101 follower arm

Filled-in example based on the public TwinWorks procedure “Replacing servomotor 6” for the open-source SO-101 arm (twinworks.com/community/so-101.html). Illustrative: check it against the official LeRobot documentation before use.

| Document ID | SOP-EX-101 | Revision | A |
| --- | --- | --- | --- |
| Owner | Lab technician | Approver | Lab lead |
| Effective date | 2026-11-04 | Next review | 2027-11-04 |
| Department / area | Robotics lab | Applies to | SO-101 follower arm, gripper |
| Supersedes | None | Status | Example |

# 1. Purpose

Replace a faulty gripper servo on the SO-101 follower arm with a new Feetech STS3215, so that the gripper opens and closes under LeRobot control again.

# 2. Scope

Applies to: the gripper servo (motor ID 6, labelled Servo6 in the 3D model) of the SO-101 follower arm.

Does not cover: the other five servos, the leader arm, or wiring faults outside the gripper servo cable.

# 3. Responsibilities

| Role | Responsibility |
| --- | --- |
| Technician | Replaces the servo, sets its ID, recalibrates the arm and fills in the log. |
| Lab lead | Checks the arm works before it is used again, and keeps this procedure current. |

# 4. Definitions and abbreviations

| Term | Definition |
| --- | --- |
| LeRobot | Hugging Face’s open-source robotics library, used to set up and calibrate the arm. |
| STS3215 | Feetech serial bus servo used in the SO-101 joints. |
| Motor ID | Address of a servo on the bus. The gripper is ID 6. |

# 5. Safety and PPE

Hazards: unexpected movement of the arm if it is powered; pinch points at the jaw.

Before you start: disconnect the power supply and the USB cable. Keep fingers clear of the jaw when you power the arm again.

PPE:  ☐ None beyond the lab rules

# 6. Materials, tools and equipment

| Item | Specification / part number | Qty |
| --- | --- | --- |
| #1 Phillips screwdriver | — | 1 |
| Feetech STS3215 servo | 1/345 gearing (C001), same voltage as the arm: 7.4 V or 12 V | 1 |
| M3 × 6 mm pan-head screws | DIN EN ISO 7045, reused | 8 |
| M2 × 6 mm screws | Servo mounting, reused | 4 |
| Computer with LeRobot installed | huggingface.co/docs/lerobot | 1 |

# 7. Procedure

| No. | Action | Key point | Reason | Image |
| --- | --- | --- | --- | --- |
| 1 | Disconnect the power supply and USB cable, then remove the four M3 × 6 mm screws on the first side of the moving jaw. | #1 Phillips screwdriver. Keep the screws: all eight are reused. | No power on the servo bus while you work. |  |
| 2 | Remove the four M3 × 6 mm screws on the other side of the moving jaw. | Support the jaw as the last screw comes out. | The jaw comes free with the last screw. |  |
| 3 | Lift the moving jaw off the servo horns and set it aside with its eight screws. | Note which way the jaw faces. | It must go back the same way. |  |
| 4 | Unplug the 3-pin cable from servomotor 6. | Pull the connector, not the wires. Servomotor 6 has one cable only, coming from servomotor 5. | It is the last servo in the chain. |  |
| 5 | Remove the four M2 × 6 mm screws that hold servomotor 6 in the lower jaw, slide the old servo out and fit the new one. | Same type: STS3215 with 1/345 gearing. Fit both horns; the top horn takes an M3 × 6 mm screw. Output shaft on the moving-jaw side. | Every joint of the follower arm uses this servo, and the jaw mounts on its horns. |  |
| 6 | Give the new servo ID 6, then plug in the cable from servomotor 5. | Connect the driver board to the new servo alone and run lerobot-setup-motors. Press Enter at the gripper prompt. | A new servo comes set to ID 1; LeRobot expects the gripper on ID 6. |  |
| 7 | Place the moving jaw back on the new servo's horns. | Facing the same way as before, screw holes lined up on both sides. | So the screws go in without force. |  |
| 8 | Refit the four M3 × 6 mm screws on the first side. | #1 Phillips screwdriver. Leave them slightly loose. | So the second side lines up. |  |
| 9 | Refit the other four screws, tighten all eight evenly, then recalibrate the arm. | Open and close the jaw by hand: it must move freely. Run lerobot-calibrate with the same arm name as before. | A new servo has no calibration. |  |

# 8. Quality checks

| Check | Method / tool | Acceptance criterion | Record |
| --- | --- | --- | --- |
| Jaw moves freely | Open and close by hand | Full travel, no binding | Tick in the log |
| Gripper servo on ID 6 | lerobot-setup-motors | “'gripper' motor id set to 6” | Log entry |
| Arm calibrated | lerobot-calibrate | Every joint moved through its full range | Log entry |

# 9. Records

| Record | Where it is kept | Retention |
| --- | --- | --- |
| Maintenance log entry: date, arm name, servo replaced, done by | Lab logbook | Life of the arm |

# 10. References

•	TwinWorks public procedure “Replacing servomotor 6”: twinworks.com/community/so-101.html

•	LeRobot SO-101 guide: huggingface.co/docs/lerobot/so101

•	SO-ARM100 design files and parts list: github.com/TheRobotStudio/SO-ARM100

# 11. Revision history

| Rev. | Date | Description of the change | Author | Approved by |
| --- | --- | --- | --- | --- |
| A | 2026-11-04 | First issue (example) | TwinWorks | — |

# Approval

|  | Prepared by | Reviewed by | Approved by |
| --- | --- | --- | --- |
| Name |  |  |  |
| Signature |  |  |  |
| Date |  |  |  |